| Course Name: | Digital Design Laboratory | Semester: | III |
| --- | --- | --- | --- |
| Date of Performance: | __24_ / __07_ / __2023____ | Batch No: | B-1 |
| Faculty Name: |  | Roll No: | 16010122104 |
| Faculty Sign & Date: |  | Grade/Marks: | ___/25 |

Experiment No: 2

Title: Binary Adders and Subtractors

Aim and Objective of the Experiment:

To implement half and full adder–subtractor using gates and IC 7483

COs to be achieved:

CO2: Use different minimization technique and solve combinational circuits.

Tools used:

Trainer kits

Theory:

Adder: The addition of two binary digits is the most basic operation performed by the digital computer. There are two types of adder: - Half adder - Full adder Half Adder: Half adder is a combinational logic circuit with two inputs and two outputs. It is the basic building block for the addition of two single-bit numbers. Full adder: A half adder has a provision not to add a carry coming from the lower order bits when multi-bit addition is performed. for this purpose, a third input terminal is added and this circuit is to add A, B, and C where A and B are the nth order bits of the number A and B respectively and C is the carry generated from the addition of (n-1) order bits. This circuit is referred to as full adder. Subtractor: Subtraction of two binary digits is one of the most basic operations performed by digital computer .there are two types of subtractors: - Half subtractor - Full subtractor Half subtractor: Logic circuit for the subtraction of B from A where A,B are 1 bit numbers is referred to as half subtract or .the subtract or process has two input and difference and borrow are the two outputs. Full subtractor: As in the case of the addition using logic gates, a full subtractor is made by combining two half-sub tractors and an additional OR-gate. A full subtractor has the borrow in capability (denoted as BORIN) and so allows cascading which results in the possibility of multi-bit subtraction. IC 7483 For subtraction of one binary number from another, we do so by adding 2’s complement of the former to the latter number using a full adder circuit. IC 7483 is a 16 pin, 4-bit full adder. This IC has a provision to add the carry output to transfer and end around carry output using Co and C4 respectively. 2’s complement: 2’s complement of any binary no. can be obtained by adding 1 in 1’scomplement of that no. e.g. 2’s complement of +(10)10 =1010is 1C of 1010; 0101. +; 1. -(10)10; 0110. In 2’s complement subtraction using IC 7483, we are representing negative number in 2’s complement form and then adding it with 1st number. Implementation Details: Half Adder Block Diagram Half Adder Circuit Truth Table for Half Adder Inputs; Outputs. A; B; S; C. 0; 0; 0; 0. 0; 1; 1; 0. 1; 0; 1; 0. 1; 1; 0; 1. From the truth table (with steps): S=AB’+A’B C=A.B Inputs; Outputs. A; B; A ⊕ B; A . B; S; C. 0; 0; 0; 0; 0; 0. 0; 1; 1; 0; 1; 0. 1; 0; 1; 0; 1; 0. 1; 1; 0; 1; 0; 1. Full Adder Block Diagram Full Adder Circuit Truth Table for Full Adder Sr. No; A; B; Cin; S; Cout. 1.; 0; 0; 0; 0; 0. 2.; 0; 0; 1; 1; 0. 3.; 0; 1; 0; 1; 0. 4.; 0; 1; 1; 0; 1. 5.; 1; 0; 0; 1; 0. 6.; 1; 0; 1; 0; 1. 7.; 1; 1; 0; 0; 1. 8.; 1; 1; 1; 1; 1. From the truth table (with steps): S = A’B’Cin + A’BCin’ + AB’Cin’ + ABCin Cout = ACin + AB+ BCin Inputs; Operations; Ouputs. A; B; Cin; A ⊕ B = D; D ⊕ Cin = E; D . Cin = F; A . B = G; F + G = H; S; C. 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. 0; 0; 1; 0; 1; 0; 0; 0; 1; 0. 0; 1; 0; 1; 1; 0; 0; 0; 1; 0. 0; 1; 1; 1; 0; 1; 0; 1; 0; 1. 1; 0; 0; 1; 1; 0; 0; 0; 1; 0. 1; 0; 1; 1; 1; 1; 0; 1; 0; 1. 1; 1; 0; 0; 0; 0; 1; 1; 0; 1. 1; 1; 1; 0; 1; 0; 1; 1; 1; 1. Half Subtractor Block Diagram Half Subtractor Circuit Truth Table for Half Subtractor A; B; DIFFERENCE(D); BORROW(Bo). 0; 0; 0; 0. 1; 0; 1; 0. 0; 1; 1; 1. 1; 1; 0; 0. From the truth table (with steps) : Difference (D) = A’B + AB’ Borrow(B) = A’B Inputs; Outputs. A; B; ~A = C; A ⊕ B = E; B . C = F; D; Borrow. 0; 0; 1; 0; 0; 0; 0. 0; 1; 1; 1; 1; 1; 1. 1; 0; 0; 1; 0; 1; 0. 1; 1; 0; 0; 0; 0; 0. Full Subtractor Block Diagram Full Subtractor Circuit Truth Table for Full subtractor A; B; BIN; D; BOROUT. 0; 0; 0; 0; 0. 0; 0; 1; 1; 1. 0; 1; 0; 1; 1. 0; 1; 1; 0; 1. 1; 0; 0; 1; 0. 1; 0; 1; 0; 0. 1; 1; 0; 0; 0. 1; 1; 1; 1; 1. From the truth table (with steps): Difference = A’B’Bin + A’BBin’ + AB’Bin’ + ABBin Borrow out= A’B + A’Bin + BBin Input; Output. A; B; Bin; A ⊕ B = C; ~A = E; E.B = F; Bin⊕ C = G; ~C = F; F.Bin = H; F + H = I; D; Borrout. 0; 0; 0; 0; 1; 0; 0; 1; 0; 1; 0; 0. 0; 0; 1; 0; 1; 0; 1; 1; 1; 1; 1; 1. 0; 1; 0; 1; 1; 1; 1; 0; 0; 0; 1; 1. 0; 1; 1; 1; 1; 1; 0; 0; 0; 0; 0; 1. 1; 0; 0; 1; 0; 0; 1; 0; 0; 0; 1; 0. 1; 0; 1; 1; 0; 0; 0; 0; 0; 0; 0; 0. 1; 1; 0; 0; 0; 0; 0; 1; 0; 1; 0; 0. 1; 1; 1; 0; 0; 0; 1; 1; 1; 1; 1; 1. Example: 1) 710 -210 = 510. 7; 0111. 2; 0010. 1’C of 2; 1101. + 1. 2’C of 2; 1110. 0111 + 1110 1 0101 Pin Diagram IC7483 Adder Subtractor

Implementation Details

Procedure: - Locate the IC 7483 and 4-not gates block on trainer kit. - Connect 1st input no. to A4-A1 input slot and 2nd (negative) no. to B4-B1 through 4-not gates (1C of 2nd no.) - Connect high input to Co so that it will get added with 1C of 2nd no. to get 2C. - Connect 4-bit output to the output indicators. - Switch ON the power supply and monitor the output for various input combinations.

Post Lab Subjective/Objective type Questions:

- Design a full adder using two half adders. - Perform the following Binary subtraction with the help of appropriate ICs: - 6-4

Conclusion:

Circuits of binary adder and sub tractors were studied on the IC kit using . .Connectors and tested using sample values.

Signature of faculty in-charge with Date: