Experiment / Assignment / Tutorial No. 1

Grade: AA / AB / BB / BC / CC / CD /DD

Signature of the Staff In-charge with date

Batch: B-1 Roll No.: 16010122104 Experiment / assignment / tutorial No.: 1

Title: Basic Gates & Universal Gates

__________________________________________________________________________________

Objective: To study the basic gates: AND, OR, NOT and universal gates: NAND, NOR, XOR, XNOR

__________________________________________________________________________

Expected Outcome of Experiment:

CO1: Recall basic gates and binary, octal & hexadecimal calculations and conversions.

__________________________________________________________________________

Books/ Journals/ Websites referred:

- Vlab Link: http://vlabs.iitkgp.ernet.in/dec/#

- R. P. Jain, “Modern Digital Electronics”, Tata McGraw Hill

- http://www.ee.surrey.ac.uk/Projects/Labview/gatesfunc/

- http://www.electronics-tutorials.ws/boolean/bool_6.html

Pre Lab/ Prior Concepts:

Gate is a logic circuit with one or more inputs but only one output. Gates are digital (two state) circuit because the input & output are either low or high. Gates provide high output for certain combinations of input & for other combinations the output is low. Total number of combinations for a gate is 2^n; where n is number of input.

Classification: The two types of gate are:

- Basic or Fundamental Gates:

- Derived Gates:

Symbols of gates

| Type of IC | Specification |
| --- | --- |

Implementation Details:

Basic Gates

- OR gate: The OR gate has two or more inputs but only 1 output. If any or all the inputs are high, the output is high. If all the inputs are low, the output is low.

Y= A + B

Symbol for OR gate	                     Pin Diagram For IC 7432

The truth table for OR operations are:

- AND gate: The AND gate has two or more inputs but only one output. If any or all inputs are high then output is also high

Y = A.B

Symbol for AND gate	           Pin Diagram For IC 7408

The truth table for AND operations are:

- NOT gate: The Not gate is a gate with only one input and one output. The output is always in opposite state of an input. A NOT gate is also called as Inverter because it performs inversion.

Y= A’

Symbol for NOT gate	Pin Diagram For IC 7404

The truth table for NOT operations is:

Derived Gates/Universal Gates

  - NAND gate

  - NOR gate

  - EX-OR gate

  - EX-NOR gate

- NAND gate: This is a NOT-AND gate which is equal to an AND gate followed by a NOT gate. The outputs of all NAND gates are high if any of the inputs are low. The symbol is an AND gate with a small circle on the output. The small circle represents inversion.

Y= (A.B)’

The truth table for NAND operations is:

- NOR gate: This is a NOT-OR gate which is equal to an OR gate followed by a NOT gate. The outputs of all NOR gates are low if any of the inputs are high. The symbol is an OR gate with a small circle on the output. The small circle represents inversion.

Y= (A+B)’

The truth table for NOR operations are:

- EX-OR gate: The 'Exclusive-OR' gate is a circuit which will give a high output if either, but not both, of its two inputs are high. An encircled plus sign ( ) is used to show the EX-OR operation

Y=A⨁

B

Symbol for Ex-OR gate	Pin Diagram For IC 7486

The truth table for XOR operations is:

- EX-NOR gate: The 'Exclusive-NOR' gate circuit does the opposite to the EOR gate. It will give a low output if either, but not both, of its two inputs are high. The symbol is an EXOR gate with a small circle on the output. The small circle represents inversion

Y=(A⨁

B)’

The truth table for XNOR operations is:

Implementation Using NAND Gate

NOT GATE								        STEPS

A NOT gate is made by joining the inputs of a NAND gate together. Since a NAND gate is equivalent to an AND gate followed by a NOT gate, joining the inputs of a NAND gate leaves only the NOT gate.

AND GATE

An AND gate is made by inverting the output of a NAND gate as shown below.

OR GATE

If the truth table for a NAND gate is examined or by applying De Morgan's Laws, it can be seen that if any of the inputs are 0, then the output will be 1. To be an OR gate, however, the output must be 1 if any input is 1. Therefore, if the inputs are inverted, any high input will trigger a high output.

IMPLEMENTATION USING NOR GATE

NOT GATE			        						STEPS

This is made by joining the inputs of a NOR gate. As a NOR gate is equivalent to an OR gate leading to NOT gate, this automatically sees to the "OR" part of the NOR gate, eliminating it from consideration and leaving only the NOT part.

AND GATE

An AND gate is made by inverting the output of a NAND gate as shown below.

OR GATE

If the truth table for a NAND gate is examined or by applying De Morgan's Laws, it can be seen that if any of the inputs are 0, then the output will be 1. To be an OR gate, however, the output must be 1 if any input is 1. Therefore, if the inputs are inverted, any high input will trigger a high output.

Conclusion:

Post Lab Descriptive Questions

- Verify the expression (A∙B)' + C by:

- Using NAND Gate directly.

- Using AND & NOT gate consecutively.

- Implement the following expressions using a combination of gates:

- (A'+B)∙B

- (A∙B)+A'

- A∙ (B∙B')

- (A'⊕B)∙A